# Lab 7: Network Traffic Analysis

## Lab Overview

In this lab, you will use a web-based simulator to understand how packets flow across a local area network, and then use Wireshark to capture and analyze real network traffic on your local machine.

Please download the latest version of Wireshark here: https://www.wireshark.org/download.html

Helpful Resources for the lab:

- NetLab Documentation: https://netlab.thecybersecguru.com/docs

- NetLab Blog / Updates: https://netlab.thecybersecguru.com/blog

- Wireshark Documentation: https://www.wireshark.org/docs/

- Socket library documentation: https://docs.python.org/3/library/socket.html

Screenshot Formatting & Visual Annotations

To receive full credit for this lab, your screenshots must demonstrate that you have found the specific data being requested. For the deliverables below, you will use visual annotations:

- Red box: Use the annotate tool in MS Word to draw a clear red box outlining the general section or packet row being analyzed.

- Yellow highlight: Use a yellow highlight tool to answer the prompt by highlighting the exact specific criteria requested (e.g., highlighting only the destination MAC address, or only the specific IP of the top talker).

Note: Please refer to the assignment submission guidelines and Lab 0 found on the Canvas page for further details to avoid losing points.

## Part 1: ARP Analysis in Your Network

Objective:

Capture and analyze ARP traffic on your own local network using Wireshark.

Tasks:

- Step 1 – Start Wireshark on your active network interface.

- Step 2 – Determine your machine's current IP address using terminal/command prompt: ifconfig on macOS/Linux, or ipconfig on Windows.

- Step 3 – Determine your default gateway.

- Step 4 – Generate ARP traffic by pinging your default gateway or another valid IP on your local subnet. In terminal/command prompt type ping x.x.x.x on macOS/Linux and Windows. (CTRL/CMD + C stops the pings).

Instructor Note: If the ping times out or fails, that is okay! Many modern routers and computers block ICMP (ping) traffic for security. The ARP request is still sent before the ping, so proceed to the next step and check Wireshark regardless.

- Step 5 – Use the Wireshark filter: arp

Deliverables:

- Full screenshot of your terminal/command prompt. Draw a red box around your active network adapter's details and yellow highlight the specific IPv4 address assigned to your machine. (3 points)

- Full screenshot showing the ping command to your default gateway. Draw a red box around the output and yellow highlight the IP address that is replying (or timing out). (3 points)

- Full screenshot showing one ARP Request packet. Draw a red box around the packet details pane (middle window) and yellow highlight the specific "Destination" MAC address that proves this packet is a broadcast to the entire network. (3 points)

- Full screenshot of the ARP Reply packet. Draw a red box around the Info column and yellow highlight the specific MAC Address being returned to your machine. (3 points)

Analysis Questions:

- Q1. Explain what ARP is used for in networking and why it is only used in the local network. (2 points)

- Q2. Explain how an on-path attacker can take advantage of ARP to redirect traffic from a victim machine to their own machine. (2 points)

- Q3. What defenses can we implement in a network to help prevent attacks on ARP? Explain how that defense helps mitigate attacks on ARP. (2 points)

## Part 2: Visualizing Packets in a Web Simulator

Objective:

Use a free web-based network simulator to load a pre-built Local Area Network (LAN) and visually observe the flow of ARP and ICMP packets. This will help you understand the basics of networking before looking at raw Wireshark data.

(For a deeper dive into the simulator's capabilities, check out the NetLab Documentation).

Tasks:

- Step 1 – Open a modern web browser and navigate to NetLab (https://netlab.thecybersecguru.com). Wait for the simulator workspace to load.

- Step 2 – In the top right navigation bar, click the Templates & Snapshots icon (it looks like three connected squares).

- Step 3 – A menu will appear with "Starter Templates". Click the Load Template button under the Starter LAN option.

- Step 4 – You will now see a pre-built topology with a Router (2911), a Switch (2960-24TT), a PC (PC-PT), and a Server (Server-PT).

- Step 5 – Click on the Server-PT device, open its network configuration/properties, and find its assigned IPv4 Address. Note this down.

- Step 6 – Click on the PC-PT device and open its terminal/command prompt tool.

- Step 7 – Type ping [Server IP Address] (using the IP you found in Step 5) and press Enter.

- Step 8 – Watch the simulation animation on the main workspace. You should see an initial broadcast (the ARP request) go out to find the MAC address, followed by the direct ICMP (ping) packets moving back and forth between the PC and the Server.

Deliverables:

- Full screenshot of your NetLab web browser showing the connected "Starter LAN" network topology. Draw a red box around the 2960-24TT Switch and yellow highlight the green link indicators connecting it to the PC and Server. (3 points)

- Full screenshot of the simulation event log/terminal showing the successful traffic. Draw a red box around the ping output and yellow highlight the specific packet type (ARP) that had to be sent before the ping could succeed. (3 points)

Analysis Questions:

- Q4. When you initiated the ping in the simulator, why did the network send an ARP packet before sending the ICMP (ping) packet? (2 points)

- Q5. Based on the visual simulation, what did the Switch do when it received the very first ARP request from the PC? Did it send it to just the intended destination, or all connected devices? (2 points)

- Q6. How does viewing packets visually in a web simulator help you understand what you are looking at in Wireshark? (2 points)

## Part 3: TELNET Traffic Analysis

Objective:

Observe insecure plaintext communication using Wireshark and embed your unique identifier into the captured traffic.

Tasks:

- Step 1 – Start Capture: Open Wireshark on your local machine, select your active network interface (e.g., Wi-Fi: en0), and click Start Capture.

- Step 2 – Run TELNET: In Terminal/Command prompt, run: telnet telehack.com 23.

Instructor Note:

• Mac users: Run brew install telnet in the terminal first to install the tool.

• Windows users: Telnet is disabled by default. You may need to enable the Telnet Client via "Turn Windows features on or off" in the Control Panel, or use an alternative terminal emulator like PuTTY.

- Step 3 – Generate Observable Traffic & Unique ID: Once connected to Telehack, type your First Name and your Student ID as a single continuous string (e.g., JaneDoe-987654321) and press Enter. The server will likely say "command not found," but that is fine—your keystrokes have been transmitted over the network.

- Step 4 – Exit TELNET by pressing Ctrl + ] and typing quit.

- Step 5 – Apply Filter in Wireshark: In the filter bar, type: tcp.port == 23 and press Enter.

- Step 6 – Inspect Data: Select any TELNET packet from your session, right-click, and select Follow → TCP Stream.

Deliverables:

- Full screenshot showing the TELNET packet list. Draw a red box around the packet row and yellow highlight the specific destination IP address of the telehack server. (3 points)

- Full screenshot showing the TCP Stream window (which displays the reassembled cleartext). Draw a red box around the block of text and yellow highlight your specific Name and Student ID that was successfully captured traversing the network in plaintext. (Note: This screenshot serves as your proof of lab completion). (3 points)

Analysis Questions:

- Q7. What is the server IP address for the TELNET connection? (2 points)

- Q8. Why can individual keystrokes appear in separate TELNET packets? (2 points)

- Q9. Based on your capture and the tasks above, why is TELNET insecure and why can it leak plaintext information? (2 points)

## Part 4: SSH Traffic Analysis

Objective:

Compare encrypted SSH traffic with TELNET.

Tasks:

- Step 1 – Start a fresh Wireshark capture.

- Step 2 – Connect to new@sdf.org using SSH in terminal/command prompt.

- Step 3 – In Wireshark apply the filter tcp.port == 22.

Instructor Note: When looking for the SSH negotiation/key exchange phase for your deliverables, look at the Protocol column. The packets will say SSHv2, and the Info column will typically mention Key Exchange Init or Diffie-Hellman.

Deliverables:

- Full screenshot showing the SSH connection in terminal/command prompt. Draw a red box around the terminal and yellow highlight the command used to initiate the secure connection. (3 points)

- Full screenshot showing the data portion of an SSH packet in the details pane. Draw a red box around the payload section and yellow highlight the encrypted, unreadable gibberish data. (3 points)

- Full screenshot showing the SSH negotiation/key exchange phase in the packet list. Draw a red box around the specific packet and yellow highlight the exact protocol version being used (e.g., SSHv2). (3 points)

Analysis Questions:

- Q10. Compare the data of the SSH and TELNET packets, what differences do you notice? (2 points)

- Q11. How is SSH more secure than TELNET against eavesdropping on packets? (2 points)

- Q12. Explain the steps of the SSH negotiation/key exchange phase and why it is important to SSH connections. (2 points)

## Part 5: TCP Connection Analysis

Objective:

Analyze the fundamental "Three-Way Handshake" and sequence of a TCP connection.

Tasks:

- Step 1 – Apply the filter tcp in Wireshark.

- Step 2 – Choose one of the TELNET or SSH sessions from your previous captures.

- Step 3 – Locate the beginning of that specific session. Look for the three sequential packets that start the connection: [SYN], [SYN, ACK], and [ACK].

- Step 4 – Right-click one of these packets and select Follow → TCP Stream to see how Wireshark reassembles the conversation.

Deliverables:

- Full screenshot showing the TCP Three-Way Handshake (SYN, SYN/ACK, ACK) in sequence. Draw a red box around the three packets and yellow highlight the packet sent by the server back to the client. (3 points)

- Full screenshot of the TCP Header details in the middle Packet Details pane for the final [ACK] packet. Draw a red box around the Transmission Control Protocol section and yellow highlight the Acknowledgment Number. (3 points)

Analysis Questions:

- Q13. Describe the purpose of the SYN and ACK flags during the establishment of a connection. (2 points)

- Q14. Look at the Relative Sequence Numbers in your capture. Why does TCP use sequence numbers instead of just sending raw data? (2 points)

- Q15. What happens to the TCP connection when you typed "quit" or closed the terminal? Identify the flag used to gracefully close a TCP connection. (2 points)

## Part 6: Protocol Statistics and Network Evidence

Objective:

Use Wireshark's built-in analytical tools to summarize network traffic and identify top talkers.

Tasks:

- Step 1 – Navigate to the top menu and select Statistics → Protocol Hierarchy. This shows a breakdown and distribution of all protocols captured.

- Step 2 – Navigate to Statistics → Conversations. Click on the IPv4 tab to see which two IP addresses exchanged the most data.

Deliverables:

- Full screenshot of the Protocol Hierarchy Statistics window. Draw a red box around the protocol list and yellow highlight the specific Application Layer protocol that accounted for the highest percentage of packets. (3 points)

- Full screenshot of the IPv4 Conversations window. Draw a red box around the conversation list and yellow highlight the IP address of the machine you exchanged the most Bytes with (the "top talker"). (3 points)

Analysis Questions:

- Q16. In the Protocol Hierarchy, which protocol accounted for the majority of your traffic? Does this match your activity during the lab? (2 points)

- Q17. Why is the "Conversations" tool useful for a Network Administrator or a Security Analyst when investigating a potential data breach? (2 points)

- Q18. If you saw a large amount of "Unknown" or "Malformed" traffic in these statistics, what might that indicate about the network or the capture? (2 points)

## Part 7: Network Programming with Python Sockets

Objective:

Write a Python client script to interact with a provided server script, complete a two-way TCP exchange, and capture the custom traffic locally using Wireshark.

Tasks:

- Step 1 – Open a text/code editor and create two new files: server.py and client.py.

- Step 2 – Copy and paste the following completely written server code into server.py and save it. Notice that the server is looking for a specific keyword in your message to grant a response.

import socket import hashlib HOST = '127.0.0.1' # Standard loopback interface address (localhost) PORT = 65432 # Port to listen on (non-privileged ports are > 1023) with socket.socket(socket.AF_INET, socket.SOCK_STREAM) as s: s.bind((HOST, PORT)) s.listen() print(f"Server is listening on {HOST}:{PORT}...") conn, addr = s.accept() with conn: print(f"Connection established with {addr}") data = conn.recv(1024).decode() print(f"Received from client: {data}") # Server logic check if "robux" in data.lower(): tx_hash = hashlib.md5(data.encode()).hexdigest() response = f"SERVER_ACK: Transaction Approved. TX_ID: {tx_hash}" else: response = "SERVER_ERR: Invalid request format." conn.sendall(response.encode())

- Step 3 – Copy the following skeleton code into client.py. You must use the official Python socket documentation linked at the top of this lab to figure out how to fill in the missing lines. Your message variable must include your Student ID and the word "robux".

import socket HOST = '127.0.0.1' # The server's hostname or IP address PORT = 65432 # The port used by the server with socket.socket(socket.AF_INET, socket.SOCK_STREAM) as s: # 1. TODO: Connect to the server using the HOST and PORT variables # 2. TODO: Create a string variable named 'message' containing your Student ID and the word "robux" # 3. TODO: Encode the message string to bytes and send it to the server # 4. TODO: Receive the server's response (buffer size 1024) and assign it to a variable called 'reply' # 5. TODO: Decode and print the 'reply' variable to your terminal

- Step 4 – Open Wireshark and start a new capture on the Loopback interface (often called Adapter for loopback traffic, lo0, or Software Loopback).

- Step 5 – Apply the filter tcp.port == 65432 in Wireshark.

- Step 6 – Open two separate terminal windows. Run python server.py in the first, then run python client.py in the second.

- Step 7 – Stop your Wireshark capture. Find the packet containing your data, right-click, and select Follow → TCP Stream. You should see the client request in red and the server response in blue.

Deliverables:

- Full screenshot of your code editor showing your completed client.py script. Draw a red box around the code block you wrote and yellow highlight the line where you used the recv() method. (3 points)

- Full screenshot showing both of your terminal windows side-by-side. Draw a red box around the client terminal and yellow highlight the TX_ID hash that was successfully received from the server. (3 points)

- Full screenshot of the Wireshark TCP Stream window. Draw a red box around the captured conversation and yellow highlight the blue server response text showing your approved transaction. (3 points)

Analysis Questions:

- Q19. In your Python script, what is the purpose of a "socket" in network programming? (2 points)

- Q20. Why did we have to capture traffic on the "Loopback" interface (127.0.0.1) instead of your Wi-Fi or Ethernet adapter for this specific script? (2 points)

- Q21. You had to define a buffer size (e.g., 1024) when using the recv() function in your client. Why do network applications need to define a buffer size for incoming data? (2 points)